# Technical Meeting Minutes – OT Cybersecurity Assessment for GNV (NAPA System)

Date: 2 April 2026
Duration: 33 minutes
Participants: HWG Sababa (Irene Parodi, Federico Rella, Moustafa Ali, Marco, Fabio Morana), NAPA (Harri Ikävalko, Mika Väkiparta), Roberto Parodi

1. Network Architecture Overview
- Most installations are standalone PCs with no network connection.
- New installations include TCP/IP or serial connections with ship automation.
- No systems have internet connectivity.

2. System Configuration & Operating Systems
- Server and client run on the same local machine.
- OS: Mostly Windows 11 Pro; some older systems may run Windows 10.
- No systems run Windows 11 IoT LTSC.

3. User Access Management
- Standard: normal user + two admin accounts.
- GNV systems currently use admin-level accounts.
- Passwords provided at delivery; no forced rotation.

4. Wireless Interfaces & USB Usage
- Wireless/Bluetooth expected to be disabled.
- Some systems may still contain unused adapters.
- USB ports allowed due to users having admin rights.

5. System Hardening & Installation Procedures
- Systems delivered with hardened Windows image.
- No extra customer-side procedures documented.

6. System Updates & Patch Management
- Microsoft Defender installed; offline signatures may be outdated.
- Windows updates only applied at delivery unless service agreement exists.
- NAPA software updates covered by standard service agreements.

7. Data Exchange & Encryption
- Ship network data exchange uses TLS with self-signed certificates.

8. Cloud Capabilities – NAPA Fleet Intelligence
- Optional cloud connectivity not enabled on GNV ships.

9. Logs & Monitoring
- Windows Event Logs available locally.
- Internal debugging logs exist; no operational activity logs.

10. Requested Documentation & Next Steps
- NAPA to provide diagrams, installation lists, and serial interface details.
- HWG Sababa to review and follow up with further questions.